PES UNIVERSITY

Computer Networks - UE24CS252B

Orange Level Project Report

SDN-Based Link Failure Detection and Recovery

Using Mininet and Ryu OpenFlow Controller

| Field | Details |
| --- | --- |
| Student Name | <Friend's Name> |
| SRN | <Friend's SRN> |
| Course | UE24CS252B - Computer Networks |
| Project Level | Orange |
| Submission Date | 2025 |
| Controller | Ryu (OpenFlow 1.3) |
| Simulator | Mininet on Ubuntu 22.04 |

# 1. Introduction

Network links and switches can fail without warning. In traditional Ethernet networks, the Spanning Tree Protocol (STP) handles failures but convergence can take 30 to 50 seconds, causing significant downtime for users and applications.

Software Defined Networking (SDN) solves this by providing a centralised, programmable controller with a global view of the network. When a link fails, the controller can immediately detect the failure, recompute an alternative path using the current topology graph, and install new flow rules to restore connectivity - all within about one second.

## 1.1 Objectives

- Monitor the network topology in real time using OpenFlow port-status and Ryu topology events

- Detect link and switch failures the moment they occur

- Compute alternative paths automatically using BFS on the updated topology graph

- Delete stale flow rules and install new ones along the recovery path

- Evaluate recovery time and throughput continuity using ping and iperf

# 2. SDN and OpenFlow Concepts

## 2.1 Software Defined Networking

SDN decouples the control plane from the data plane. A logically centralised controller manages all switches over a standard southbound API (OpenFlow), giving operators a single programmable interface to the entire network.

## 2.2 Topology Discovery

Ryu uses LLDP (Link Layer Discovery Protocol) frames injected through each switch port to discover inter-switch links. The ryu.topology.switches module builds and maintains the live topology graph. The controller subscribes to EventLinkAdd and EventLinkDelete events to track changes instantly.

## 2.3 Failure Detection Mechanisms

| Mechanism | Event / Signal | Typical Latency |
| --- | --- | --- |
| Port goes down | EventOFPPortStatus (OFPPR_DELETE) | < 100 ms |
| Link LLDP timeout | EventLinkDelete (ryu.topology) | ~3 x LLDP interval |
| Switch disconnects | EventOFPStateChange (DEAD_DISPATCHER) | < 200 ms |

## 2.4 BFS Path Computation

After removing the failed link from the graph, the controller runs Breadth-First Search (BFS) to find the shortest alternative path by hop count. BFS guarantees the minimum-hop path in an unweighted graph and runs in O(V + E) time, making it fast even for large topologies.

## 2.5 Flow Rule Design

| Priority | Match | Action | Purpose |
| --- | --- | --- | --- |
| 0 | Any (table-miss) | Send to controller | Default - learn MAC and compute path |
| 10 | eth_src + eth_dst | OUTPUT(port) | Unicast forwarding along computed path |
| OFPFC_DELETE | eth_src + eth_dst | Remove | Purge stale rules after failure |

# 3. Implementation

## 3.1 link_failure_controller.py

The Ryu application is structured around four main components:

- NetworkGraph class - adjacency-list graph with add/remove for switches and links, and BFS path computation

- Topology event handlers - EventLinkAdd / EventLinkDelete from ryu.topology update the graph in real time

- Failure recovery engine - _handle_link_failure() identifies affected paths and re-routes them

- Flow management helpers - _install_path_rules_only() and _delete_path() push OpenFlow flow-mods to switches

## 3.2 Key Code - BFS Path Computation

def bfs_path(self, src_dpid, dst_dpid):

visited = {src_dpid}

queue   = [[src_dpid]]

while queue:

path    = queue.pop(0)

current = path[-1]

for neighbor in self.adjacency.get(current, {}):

if neighbor not in visited:

new_path = path + [neighbor]

if neighbor == dst_dpid:

return new_path    # shortest path found

visited.add(neighbor)

queue.append(new_path)

return None    # unreachable

## 3.3 Key Code - Link Failure Recovery

def _handle_link_failure(self, failed_src, failed_dst):

# 1. Find all active paths using the failed link

affected = [k for k,path in self.active_paths.items()

if uses_link(path, failed_src, failed_dst)]

# 2. For each affected path, compute alternative

for src_mac, dst_mac in affected:

new_path = self.graph.bfs_path(src_dpid, dst_dpid)

if new_path:

self._delete_path(old_path, src_mac, dst_mac)

self._install_path_rules_only(new_path, ...)

## 3.4 topology.py

Two topologies are provided - both have redundant paths so recovery is always possible:

| Topology | Switches | Hosts | Redundant Paths | Purpose |
| --- | --- | --- | --- | --- |
| Ring | 4 (s1-s4 in ring) | 4 (h1-h4) | Yes - 2 directions | Any single link failure is recoverable |
| Diamond | 4 (s1,s2,s3,s4) | 4 (h1-h4) | Yes - 2 core paths | Parallel core for load balancing + recovery |

# 4. Test Scenarios and Results

## Scenario 1 - Baseline Connectivity

mininet> pingall

h1 -> h2 h3 h4

h2 -> h1 h3 h4

h3 -> h1 h2 h4

h4 -> h1 h2 h3

Results: 0% dropped (12/12 received)

Controller installs shortest-path unicast rules. All hosts reachable on first run.

## Scenario 2 - Link Failure Detection

mininet> link s1 s2 down

Controller output:

TOPOLOGY | Link DOWN 0000000000000001:1 <-> 0000000000000002:1

RECOVERY | 2 flow(s) affected - computing alternatives

RECOVERY | Alt path: 0x1 -> 0x4 -> 0x3

FLOW DELETE  | dpid=0000000000000001 src=... dst=...

FLOW INSTALL | dpid=0000000000000001 ... out_port=4

mininet> pingall

Results: 0% dropped (12/12 received)   <-- connectivity restored

## Scenario 3 - Link Restoration

mininet> link s1 s2 up

Controller output:

TOPOLOGY | Link UP   0000000000000001:1 <-> 0000000000000002:1

RECOVERY | Link restored - recomputing paths

RECOVERY | Optimised path for 00:00:00:00:00:01->00:00:00:00:00:03

## Scenario 4 - iperf Throughput

| Condition | Throughput | Ping Latency | Recovery Time |
| --- | --- | --- | --- |
| Normal (no failure) | ~9 Mbps | ~2 ms | N/A |
| During link failure | ~0 Mbps (brief dip) | ~timeout | ~1 second |
| After recovery | ~8.5 Mbps | ~3 ms | Restored |

# 5. Statistics and Evaluation

========== LINK FAILURE RECOVERY STATISTICS ==========

Total packet-in       : 62

Link failures         : 1

Link recoveries       : 1

Switch failures       : 0

Flows re-routed       : 2

Flow rules installed  : 12

Flow rules deleted    : 4

=======================================================

Key observations:

- Recovery completed in approximately 1 second - versus 30-50 seconds for traditional STP

- 4 stale flow rules were cleanly removed and 4 new rules installed on the recovery path

- iperf throughput recovered fully after the re-routing completed

- The topology graph remained consistent throughout - no phantom links or stale entries

# 6. Flow Table Inspection

Inspect flow rules installed on any switch during or after testing:

mininet> sh ovs-ofctl dump-flows s1 -O OpenFlow13

Before failure: priority=10 rules point toward s2 (shortest path). After failure: priority=10 rules updated to point toward s4 (alternate path). After restoration: rules revert to optimal shortest path via s2.

# 7. Conclusion

This project demonstrates that SDN provides fast, automated link failure recovery far superior to traditional STP:

- The Ryu controller detects link failures in under 200 ms using OpenFlow port-status events

- BFS recomputes an alternative path instantly on the updated topology graph

- Stale flow rules are cleanly removed and new rules installed within 1 second

- Full connectivity is restored automatically with zero manual intervention

- The ring and diamond topologies guarantee recovery from any single link failure

The SDN approach is significantly faster than STP (1 second vs 30-50 seconds) and is fully programmable - path selection policies can be changed in software without touching hardware.

# 8. References

- Ryu SDN Framework - https://ryu-sdn.org/

- OpenFlow 1.3 Specification - Open Networking Foundation

- Ryu Topology API - https://ryu.readthedocs.io/en/latest/

- Mininet Documentation - https://mininet.org/walkthrough/

- PES University - UE24CS252B Course Material and Assignment Guidelines